4QR 03

4 Quay Row, Abercanaid, Merthyr.  CF48 1YP

HERITAGE IMPACT STATEMENT

- Summarise proposed works and why.

- Significance of the asset.

- Aspects of the asset’s value to be affected by the proposal.

- Impact of the proposal – benefits as well as harm.

The proposal is in respect of No 4 Quay Row, a small c 1820 artisan cottage, and one of a row of seven built, it is believed, between 1800 and 1830 in Upper Abercanaid alongside the then busy Glamorganshire Canal, of which no’s 1-5 are listed Grade II.  I have just purchased the house (having purchased No 3 some years previously) and find that during the ownership of the outgoing owners (1991 – 2020), little or no maintenance or upkeep of the property was undertaken.  In addition, much of the work done prior to 1991 – as with the replacing of sash windows noted in the Listing Report – was of poor quality.

The proposal is aimed at rectifying these matters and providing a dwelling acceptable to modern tastes and demands while fully respecting the building’s heritage value, both as an individual piece, and as to its place in a heritage block.  The works, both external and internal, are comprised as follows:

- Front windows – replacing four non-identical windows in the front elevation with more period-compliant box sash windows incorporating double-glazed units

- Back door.  The rear of the cottage comprises a relatively recent breeze-block extension. The rear door appears not to have been of adequate quality and has been allowed to acquire serious rot.  The proposal will be to replace this with a half glazed alternative, in hardwood/treated softwood.  A suggested door pattern is attached.

- Under advice to “stiffen” exposed beams in downstairs ceiling with wooden re-enforcement. This will enable retention of the exposed beams as well as preventing the significant – and worrying - “bounce” currently affecting the boarded timber floor above.

- Incorporating a small room for toilet and handbasin in the far rear corner of the upstairs space, along with an attached built-in wardrobe/cupboard.

- Adding an additional matching bannister rail at the top of the stairs as a safety feature to prevent residents falling.

- Replaster/board, re-skim as necessary internal walls, both up- and downstairs using lime plasters in keeping with original, final composition to be confirmed on site in consultation with planning officials.  Painting to be in pale neutral colours to maximise light, using as appropriate breathable paints.

- Installation of new shower room and kitchen elements – to be “modern”, naturally, but with a nod to the heritage aspect of the building, and therefore conveying a sense of period appropriateness.

- Upstairs wooden floor to be re-carpeted.  Lower floors are all cement.  Living area to be re-carpeted, kitchen and shower area to be tiled.

HERITAGE SIGNIFICANCE - General,

A study of the history of the Quay Row area shows it to have been of considerable significance in its heyday, with the cottages, the local industrial enterprises and the canal all integrating to form a gem of what we today call industrial archaeology.  But 200 years have seen mighty change: the industry has gone, the canal dwindled to a silty and over-grown stream.  The houses alone remain.

And for many years it seems that the houses changed little – as the photograph from the 1950 seems to show.  But in recent years, both before and after initial listing in 1975, significant changes have taken place to the Row.  No’s 6 and 7 have been amalgamated and re-developed as a substantial dwelling on twice the original footprint with a raised roofline and a non-traditional arrangement of windows (Compare relevant pictures - 4QR 06).  To such an extent that this end of the Row cannot now be a realistic part of the listing.  It can also be noted that, subsequent to the building of the new road access to Chapel Square, an opportunity was created to enable No 6/7 to have its own private access from the northern side.  The old traditional shared path along the front of the Row therefore now terminates just beyond No 5, and as the plan shows, the rest of the former traditional path is now incorporated into the private garden of No 6/7. It must therefore be noted that a principal original reason for designation given in the Listing Report is now partially misleading.  Reference is there made to; “one of the best surviving groups of early industrial workers’ housing”: but the complete re-modelling of Nos 6/7 means, surely, that a qualification is required here, as only part of the group survives.  This cannot but have some implications for the heritage value of the whole and, I contend, must result in a less than remorselessly strict attitude in assessing any minor modifications that may be suggested as part of upgrading in the rest of the Row.

Also, in terms of amalgamations, it must be remembered that No’s 1 and 2 have also been amalgamated into one dwelling, meaning that here too the original patterning of door and window apertures has been compromised.

HERITAGE SIGNIFICANCE – Aspects of Quay Row,

Front Windows.  A glance at the relevant photos will show that there is significant diversity in the style and nature of the windows now in place in the whole row, with a mixture of UPVC, modern wood and more traditional sash.  No’s 5 and 6/7 have UPVC: No 4 currently has “Magnet” style softwood windows (not all of the same pattern and age, and in poor condition): No 3 has fairly new matched wooden box sash: No 1/2 has a random mixture of wooden box sash and “magnet style” windows, all in poor condition.

Internally.  I have only had view of three of the properties, and it is clear that the internal arrangements as they stand have moved very considerably away from the original plan.  No 1/2 has been largely hollowed out in order to create a single dwelling out of the two cottages.  The two original staircases have been replaced by a single wooden one.  No 3 has also had its stone stair blocked off and a new wooden stair installed across the rear of the main room.  A significant – and very classy – modern single-storey extension is added to hold a large bathroom, two utility cupboards and a shower/bathroom.  No 4 is the least altered.  The stone stair remains.  The upper floor had been divided with non-permanent chipboard and Perspex partitions, but these are now removed.  Downstairs to the rear of the main room a modest single-storey extension in blockwork provides for a galley kitchen and a compact shower room.  Ceiling beams remain in the main downstairs room in both No 3 and No 4.  In both, the original range/fireplace has been removed and the fireplaces have been largely filled in and smooth plastered.  I have not seen inside No 5 but, observing the rear, it is obvious that a substantial two-storey extension with double the footprint of that at No 4 has been added.

In sum, then, looking at both external and internal factors, it is clear that the heritage value in detail has been very largely eroded.  One has only to compare what is here with the beautifully restored and conserved Rhyd y Car cottages at St Fagan to realise the difference.  What is here is a strong general impression created by the façade of the 5 houses, fitted finely to their location and giving some historical flavour as to how the row would once have appeared.  It would seem to me that there should be no objecting to any small adaptations that do not detract from that general sense here of “character of place” as indicated by Planning Policy Wales.

ASPECTS OF THE ASSETS VALUE TO BE AFFECTED BY THE PROPOSAL.

It is not envisaged that there will be any negative results on the asset from these modest proposals.  The general upgrading here envisaged will increase the sustainability of the asset – “preserving the building” in the words of Planning Policy Wales, making it more attractive as a residence.  This will help ensure it survives and thrives into the future as an individual unit as well as having a positive effect on the general appearance of the whole row and encourage other owners to follow suit where applicable.  The maintenance of the exposed beams in the downstairs ceiling and retention of the circular in-wall staircase, fulfil the insistence by Planning Policy Wales on the need to preserve features of historical/architectural interest.  These two items are the only items of this kind still surviving internally in No 4 Quay Row.

IMPACT OF THE PROPOSAL.

Our claim is that the impact of these proposals will be entirely positive, in the following ways:

- The design of the proposed double-glazed front-facing sash windows will be sympathetically adapted to the existing window apertures and by being set back will look more “in period”, and be more uniform, than the non-fully matching windows they replace.  The double-glazed units will help in keeping the fabric dry and warm while the ventilating system in the frames will allow control of humidity and the flow of air.  A by-product of these new windows when fitted upstairs will be to relieve safety concerns caused by the present windows as the upper window-sills are very low to the floor.

- Many of the same considerations will apply to replacing the rotting wooden rear door. Allowing a half-glassed door in a period-sensitive style will increase the light into the rear extension.  The visual impact of such a door will enhance the rear aspect of the house.  The rear of the whole row has been so altered that the replaced door will not have any adverse effect on the appearance of the whole.

- Internal re-plastering.  Much of the original plastering had already failed and been replaced – in some cases with pink plaster which has also failed.  Only on the rear wall of the downstairs room does the original plaster appear to be fairly intact.  Re-skimming, or boarding and skimming if permitted, incorporating any required insulation, will improve the stability of internal walls, as well as their neatness of appearance, and insulating qualities, without having any negative effect on the overall historical feel.  It is proposed that this work – as it moves through to completion – will be carried out in consultation with local planning officials to ensure that compliant materials are incorporated.

- Exposed beam ceiling.  Work to strengthen the exposed beams, thus allowing them to remain exposed, will leave a substantial part of the cottage’s “period” quality as intact as possible.

- Sympathetically chosen kitchen units will enhance the interior ambiance of the downstairs areas and provide the necessities of modern living.  Likewise, the slight expansion of the shower room into previous “dead” space will allow the creation of a more spacious and comfortable area for shower, toilet, and such.  Note that this is proposed for a section of the building which is a relatively recent breeze-block extension and contains no elements of any historical value.

- All internal walls to be painted in neutral and pale shades, using breathable paint, to maximise the light.  Tongue and groove ceilings in the upstairs area (modern and currently varnished) are to be regularised/stabilized and white-painted, allowing the creation of a more accurate historical and “period” feel.

- Central Heating System.  The previous heating system, now removed, involved an early-style back boiler in the lounge fireplace, exposed pipes in a duct passing across the lounge wall and up to a large water cylinder in the upstairs space – and a resulting spiders web of pipework from there throughout the house.  Mounting a “combi boiler” in the kitchen, just underneath the proposed shower room upstairs and right next to the kitchen sink and washing machine, will centralise and concentrate the hot-water system making it far more fit for purpose.  Far fewer pipes will be needed and these will be kept out of view – behind skirting boards, etc, so compromising to the least extent possible the period look and feel of the interior.

- Built in furniture etc upstairs.  Because of the age of the structure and the desire of loading the floor beams as little as possible, it has been decided to retain the single bedroom pattern upstairs - making it similar in that way to No. 3.  For ease of use, however – with the downstairs toilet being at some distance – it is proposed to create a small en-suite toilet and to “fare” such a room into the larger space by creating a built-in wardrobe/cupboard just next to it.  A view of the floor-plan will show how this arrangement fits tidily into the space.

- Additional bannister.   The existing bannister is well placed to block from falling downstairs people who approach directly across the room.  People walking along the chimney wall are not so well protected and could slip and then tumble.  Hence the proposal for a matching bannister/safety rail, as shown in the plan.

In conclusion, it is considered that the scheme proposed here is a modest one, is in accord with the principles laid down in Planning Policy Wales, that will enhance rather than detract from the position of No. 4 Quay Row as an element in a listed row of cottages of historic and architectural interest.  It is hoped  that, as a result, Listed Building Consent to these proposals will be confirmed.

Conclusion of Heritage Statement.

Amelia Williams.        1 April 2021